MATERIAL SAFETY DATA SHEET   :       DEBONDER	PAGE : O1

(O465O7-O126P -4O183576-67OO4921)!DATE OF ISSUE	! SUPERSEDES	!

[   O2/11/94_	'[ __12/15/88

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	DEBONDER

CHEMICAL FAMILY	FORMULA

N/A	X<--MIXTURE

MANUFACTURERS  NAME:

DYNA SYSTEMS A PARTSMASTER CO DIV OF NCH

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

P.O. BOX 655326

DALLAS, TEXAS 75265-5326

PREPARED BY:	! PRODUCT CODE NUMBER ! EMERGENCY TELEPHONE NUMBER

M DICKSON/CHEMIST	67OO4921	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS)  : NITROMETHANE

HAZARD	>FLAMM/IRR   ] TLV	>1OOPPM 1.   ! PEL-->1OOPPM 2.   :

STEL(TWA)* ->15OPPM      ] CAS#-->75-52-5     j	:

CHEMICAL NAME (INGREDIENTS)  : ACETONE

HAZARD	>FLAMM/IRR     TLV	>75OPPM 1.   I PEL-->75OPPM 2.

STEL(TWA)* ->1OOOPPM     | CAS#- ->67-64- 1     |	',

SECTION III - PHYSICAL DATA

DEBCNDER

(CONTINUED)

SECTION III - PHYSICAL DATA

O2

BOILING PT.(FAHRENHEIT)  |1 F

SPEC GRAVITY (H20=1)

: 1 .06

VAPOR PRESSURE (MM HG).  |N/E

COLOR    ]STRAW

VAPOR DENSITY (AIR=1)

2.1

in/a

3O

1OO%

PH

PERCENT,    VOLATILE BY   VOLUME    (%)

SOLUBILITY IN WATER VISCOSITY

KETONES

ODOR

clarity iclear

EVAPORATION RATE (BU AC = 1 )

SLIGHT NON-VISCOUS

in/e

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED) 1 F      T.C.C.

FLAMMABLE LIMITS ACETONE

I	LEL

2.15

UEL

EXTINGUISHING   MEDIA       "ALCOHOL"
<--FOAM	X<--FOAM	X<--C02

DRY X<--CHEMICAL

WATER X<--SPRAY

<--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

WEAR SELF-CONTAINED BREATHING APPARATUS AND PROTECTIVE EQUIPMENT.

UNUSUAL FIRE & EXPLOSION HAZARDS

TREAT AS AN EXTREMELY FLAMMABLE LIQUID.

NFPA HAZARD RATING (O=INSIGNIFI CANT; 1=SLIGHT;2 = MODERATE;3 = HIGH;4 = EXTREME ) : 3 <--HEALTH   4 <--FLAMMABILITY    1 <--REACTIVITY      <--SPECIAL

SECTION V - HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

EFFECTS OF OVEREXPOSURE

- ACUTE -    (SHORT TERM EXPOSURE)

VAPORS DEADEN THE SENSE OF SMELL AFTER A SHORT EXPOSURE AND CAUSES IRRITATION OF THE NOSE, EYES AND THROAT RESULTING IN SNEEZING, TEARING AND

DEBONDER

(CONTINUED)	SECTION V - HEALTH HAZARD DATA	PAGE : O3

COUGHING.  INHALATION OF VAPORS MAY CAUSE HEADACHES, DIZZINESS, NAUSEA AND UNCONSCIOUSNESS. EYE CONTACT CAUSES SEVERE IRRITATION, DISCOMFORT, PAIN, EXCESSIVE BLINKING, TEARING, REDNESS AND SWELLING OF THE CONJUNCTIVA. PROLONGED OR REPEATED SKIN CONTACT MAY CAUSE DEFATTING AND DRYING OF THE SKIN.  INGESTION MAY CAUSE HEADACHE, DIZZINESS, DROWSINESS AND VOMITING. ASPIRATION DURING INGESTION OR VOMITING CAN CAUSE LUNG DAMAGE.

- CHRONIC -  (LONG TERM EXPOSURE) NO CHRONIC HUMAN DATA GIVEN. TARGET ORGANS: LIVER AND KIDNEYS CONDITIONS AGGRAVATED BY EXPOSURE: PREEXISTING SKIN CONDITIONS.

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INGESTION   <-- ABSORPTION

EMERGENCY 8. FIRST AID PROCEDURES

INHALATION :

REMOVE FROM THE AREA TO FRESH AIR. IF BREATHING HAS STOPPED, CLEAR THE AIRWAY AND START MOUTH-TO-MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

EYE CONTACT:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES, AND CONTINUE FLUSHING THE EYES FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

SKIN CONTACT:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION_IF IRRITATION DEVELOPS.

INGESTION :

IMMEDIATELY GIVE 3 TO 4 GLASSES OF WATER AND INDUCE VOMITING BY PLACING A FINGER OR BLUNT OBJECT ON THE BACK OF THE VICTIM'S TONGUE. GIVE FLUIDS UNTIL VOMITUS IS CLEAR. GET IMMEDIATE MEDICAL ATTENTION. DO NOT INDUCE VOMITING OR GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

NOTES TO PHYSICIAN :

N/A

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC   <--YES  NTP    <--YES  OSHA    <--YES  ACGIH   <--YES  OTHER   <--YES
X<--NO         X<--NO	X<--NO	X<--NO	X<--NO

PRODUCT

IHL-RAT	LC5O:	<32,OOO PPM/4H 4.

SKN-RBT	LD5O:	<3.2 G/KG 4.

ORL-RAT	LD5O:	121O MG/KG 4.

NITROMETHANE

ORL-RAT	LD5O:	94O MG/KG 3.

IHL-MKY	LCLO:	1OOO PPM 3.

IVN-RBT	LDLO:	75O MG/KG 3.

DEBONDER

(CONTINUED)	SECTION VI - TOXICITY INFORMATION	PAGE : O4

ACETONE

ORL-MAN	TDLO: 2857 MG/KG 3.

IHL-RAT	LCLO:  16OOO PPM/4H 3.

EYE RET	395O UG SEV 3.

SECTION VII - REACTIVITY DATA

|X<--STABLE     <--UNSTABL |  CONDITIONS TO AVOID

STABILITY

CONTACT WITH FLAMES AND SPARKS.

INCOMPATIBILITY (MATERIALS TO AVOID)

NITRIC, ACETIC AND SULFURIC ACID, ALUMINUM CHLORIDE PLUS ORGANIC

MATTER, POTASSIUM, SODIUM AND CALCIUM HYDROXIDE, ORGANIC AMINES.

HAZARDOUS DECOMPOSITION PRODUCTS OXIDES OF CARBON.

WILL NOT         MAY    CONDITIONS TO AVOID

polymerization|n/a

HAZARDOUS      X <--OCCUR      <--OCCUR

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

VENTILATE AREA AND REMOVE SOURCES OF IGNITION. COLLECT AS HAZARDOUS WASTE AND DILUTE IN FUEL OIL. WEAR PROTECTIVE CLOTHING AND RESPIRATORY PROTECTION WHILE CLEANING UP SPILL.

WASTE DISPOSAL METHOD

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS1

NEUTRALIZING AGENT N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

REQUIRED VENTILATION

LOCAL EXHAUST IS RECOMMENDED TO PROTECT WORKERS FROM EXPOSURE TO

LEVELS ABOVE PERMISSIBLE EXPOSURE LIMITS.